# Introduction

# uProc1 Requirements

| ID | Requirement Text | Links |
| --- | --- | --- |
| FPGA1-1 | When FPGA1 receives data on the Data 1 bus, it shall assert the Data1_Interrupt and make the data available in the Data1 register. | SRD-1 |
| FPGA1-2 | When a new value for Data1 is written to the Data1 Tx register, FPGA1 shall output the data on the Data1 bus. | SRD-2 |
| FPGA1-3 | When the value of Discrete1 changes FPGA1 shall assert the FPGA_Discrete_interrupt and make the new value available in the FPGA1 discrete status registers. | SRD-3 |
| FPGA1-4 | When a new value for Discrete2 is written to the FPGA1 discrete status registers, FPGA shall set the Discrete as specified. | SRD-4 |