# Introduction

# Reading and Writing Discrete Inputs and Outputs

FPGA1 monitors the status of the Discrete inputs. When they change it asserts and interrupt to uProc1, which then reads the value of all discretes and maintains a local copy of the status. The status is returned to the user whenever the REST API is queried (SRD-3).

In the reverse direction, when a discrete value is set via the REST API, uProc 1 wites to the relevant register (As defined in the Internal ICD) using the SPI bus to FPGA1 (SRD-4).

# Reading and Writing to Databus1.

When data is received on Databus1, FPGA1 writes the data to the register defined in the internal ICD and asserts the data interrupt to uProc1. When uProc1 receives the interrupt, it reads the data, maintains a local copy and provides it in any REST API queries (SRD-1)

When data for databus 1 is written to the REST API, uProc 1 writes the value to the FPGA register defined in the internal ICD using the SPI Bus. FPGA1 then updates the output value (SRD-2).